ISYE435/535 Exp Design For Engineering

Assignment 1

Due date: Friday, March 6, 2026

Prof.: Reinaldo Moraga – T.A.: Ibrahim Oyeyinka

Instructions: This is a group assignment. WE NEED TO SEE YOUR WORK! You MUST provide assumptions, formulas, graphs, flowcharts, and reasoning for each situation. The format to provide an answer to each given problem is MS Word. NO scanned handwritten image of any type is allowed. See syllabus instructions for submissions of group assignments. PUT YOUR NAME ON ALL PAGES AND NUMBER EACH PAGE. Discuss the statistical model for each of the problems before showing your solution.

Problem 1 (50 points): A manufacturer of television sets is interested in the effect on tube conductivity of four different types of coating. A completely randomized experiment is conducted, and the following conductivity measurements are obtained:

| Coating Type | Conductivity |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 143 | 141 | 150 | 146 |
| 2 | 152 | 149 | 137 | 143 |
| 3 | 134 | 136 | 132 | 127 |
| 4 | 129 | 127 | 132 | 129 |

(a) Is there a difference in conductivity due to coating type? Use α = 0.05.

(b) Estimate the overall mean and the treatment effects.

(c) Compute a 95 percent confidence interval estimate of the mean of coating type 4. Compute a 99 percent confidence interval estimate of the difference in means between coating types 1 and 4.

(d) Test all pairs of means with α = 0.05.

(e) Which coating type produces the highest conductivity?

(f) Assuming that coating type 4 is currently in use, what are your recommendations to the manufacturer? We wish to minimize the variability in conductivity.

(g) Analyze the residuals and draw conclusions about model adequacy.

Problem 2 (50 points): The effect of five different ingredients (A, B, C, D, E) on the reaction time of a chemical process is being studied. Each batch of new material is only large enough to permit five runs to be made. Furthermore, each run requires approximately 112 hours, so only five runs can be made in one day. The experimenter decides to run the experiment as a Latin square so that day and batch effects may be systematically controlled. She obtains the data that follow. Analyze the data from this experiment (use α = 0.05) and draw conclusions.

|  | Day | Day | Day | Day | Day |
| --- | --- | --- | --- | --- | --- |
| Batch | 1 | 2 | 3 | 4 | 5 |
| 1 | A = 8 | B = 7 | D = 1 | C = 7 | E = 3 |
| 2 | C = 11 | E = 2 | A = 7 | D = 3 | B = 8 |
| 3 | B = 4 | A = 9 | C = 10 | E = 1 | D = 5 |
| 4 | D = 6 | C = 8 | E = 6 | B = 6 | A = 10 |
| 5 | E = 4 | D = 2 | B = 3 | A = 8 | C = 8 |